NAVIS

# ESP32 Firmware & Hardware Integration

ESP32-CAM, OV2640, VL53L0X, Wi-Fi and telemetry

Version 1.0 | 31 October 2026

Architecture baseline: laptop-hosted AI processing; hybrid smartphone/wearable audio; Wi-Fi camera and sensor communication.

# 1. Firmware scope

Firmware is responsible for hardware initialization, camera capture, ToF measurement, Wi-Fi connectivity, image upload, sensor telemetry and device health. It must not run YOLO26 or Nemotron.

# 2. Firmware project structure

navis-firmware/
  src/main.cpp
  src/camera_manager.cpp + camera_manager.h
  src/tof_manager.cpp + tof_manager.h
  src/wifi_manager.cpp + wifi_manager.h
  src/api_client.cpp + api_client.h
  src/device_config.h
  platformio.ini
  README.md

# 3. Boot and runtime sequence

Boot ESP32.

Initialize camera and verify capture.

Initialize I2C and VL53L0X; report invalid state if initialization fails.

Connect to configured Wi-Fi.

Register device with laptop backend.

Start independent camera and sensor tasks.

Send periodic heartbeat and status.

Reconnect with bounded backoff after network failure.

# 4. Candidate wiring

| VL53L0X pin | Candidate ESP32-CAM connection | Notes |
| --- | --- | --- |
| VIN | Suitable regulated supply | Verify breakout voltage requirements. |
| GND | GND | Common ground. |
| SDA | GPIO13 candidate | Check exact board; shared with microSD interface on common boards. |
| SCL | GPIO14 candidate | Check exact board; shared with microSD interface on common boards. |

This is not universal wiring. Confirm the exact ESP32-CAM board revision, camera pin usage, microSD configuration, boot strapping and logic-level compatibility before connecting. Disable microSD if using its pins for I2C.

# 5. Initial runtime settings

| Task | Initial target |
| --- | --- |
| ToF telemetry | 10 Hz, tune after sensor timing tests |
| Camera capture | On demand or 1–2 FPS for prototype |
| Heartbeat | Every 5 seconds |
| Reconnect check | Every 2–5 seconds |
| Frame format | JPEG |
| Initial image size | VGA or another measured configuration |

# 6. Data contracts

## 6.1 Sensor telemetry

{
  "device_id": "navis-esp32-001",
  "sensor_id": "tof-001",
  "distance_mm": 650,
  "valid": true,
  "timestamp_ms": 1793440000
}

## 6.2 Camera upload

POST /api/v1/camera/frame as multipart/form-data with device_id, timestamp_ms and JPEG frame.

# 7. Firmware reliability

Keep sensor task independent of camera upload task.

Validate sensor status and reject invalid ranging values.

Use bounded reconnect backoff and do not block sensor reads indefinitely on HTTP.

Use a stable power supply and monitor brownout/reset logs.

Avoid storing captured frames on flash unless explicitly required.

Report camera and sensor failure independently.

# 8. Hardware acceptance tests

Camera initializes repeatedly after cold boot.

Valid JPEG reaches laptop over Wi-Fi.

VL53L0X produces stable readings against test targets.

Sensor telemetry continues while image uploads occur.

Wi-Fi interruption triggers recovery and status update.

No microSD/GPIO conflict or boot failure occurs.

Power and thermal behaviour are tested on the spectacle assembly.