NAVIS

Technical Requirements Document (TRD)

SAARTHI 2026 Hackathon | Version 1.0 | 29 October 2026

# 1. Purpose and technical summary

This TRD specifies the system architecture, module boundaries, runtime behavior, API contracts, repository layout, hardware setup, networking, implementation tasks, testing and deployment for the NAVIS SAARTHI 2026 hackathon prototype.

| Decision | Specification |
| --- | --- |
| Wearable | ESP32-CAM with OV2640; Wi-Fi camera stream/snapshot. |
| Compute | Laptop; Python 3.12 virtual environment. |
| Backend | FastAPI + Uvicorn; REST + WebSocket. |
| Detection | Ultralytics YOLO small/nano model; model choice validated on laptop. |
| Image handling | OpenCV; latest-frame sampling; bounded buffers. |
| OCR | PaddleOCR or Tesseract; choose one during first 4 hours. |
| Mobile | Native Android Kotlin + Jetpack Compose. |
| Speech | Android SpeechRecognizer and TextToSpeech. |
| Location | Android location APIs plus a pedestrian routing service. |
| Persistence | Environment configuration, app preferences and JSONL event logs. |

# 2. Logical architecture and data flow

Firmware layer: initialize camera, join Wi-Fi, expose stream/snapshot and log network/camera failures.

Camera adapter: connect to the stream, decode JPEG frames, timestamp them and keep a bounded latest-frame buffer.

Inference module: run the selected YOLO model on sampled frames and normalize output to class, confidence, bounding box, image position and timestamp.

Hazard policy module: apply confidence threshold, temporal persistence, stale-event checks and duplicate cooldown; assign priority and message.

OCR/scene modules: execute only when requested initially, to preserve compute and network capacity.

Event manager: broadcast normalized JSON events to connected Android clients over WebSocket.

Android network layer: call REST endpoints, subscribe to WebSocket, reconnect with bounded backoff and surface connection state.

Android interaction layer: recognize commands, dispatch intents, speak events and keep navigation instructions separate from object narration.

Navigation layer: obtain location, resolve destination, request pedestrian route and announce maneuvers only when route/location data is valid.

# 3. Repository layout

navis/
  README.md
  .env.example
  .gitignore
  firmware/
    esp32_cam/
      esp32_cam.ino
  backend/
    requirements.txt
    app/
      main.py
      camera.py
      detection.py
      hazard_engine.py
      ocr.py
      commands.py
      events.py
      settings.py
      schemas.py
    tests/
      test_hazard_engine.py
      test_api.py
  android/
    app/
      src/main/java/.../
        ui/
        voice/
        location/
        network/
        settings/
        qr/
  docs/
    architecture.md
    api-contract.md
    wiring.md
    test-plan.md
  data/
    test-scenarios/
    evaluation/

# 4. Backend environment and startup

Use Python 3.12 for the hackathon environment unless a required library forces a different tested version. Create a virtual environment and pin working package versions before integration.

cd navis/backend
python -m venv .venv

# Windows PowerShell
.venv\Scripts\Activate.ps1

# macOS/Linux
source .venv/bin/activate

pip install fastapi uvicorn[standard] opencv-python ultralytics python-multipart pydantic-settings
pip freeze > requirements.txt

uvicorn app.main:app --host 0.0.0.0 --port 8000 --reload

Keep optional OCR and cloud-VLM dependencies separate until the core camera-to-audio pipeline is working. Never commit API keys or Wi-Fi passwords.

# 5. Configuration

| Variable | Meaning | Example / rule |
| --- | --- | --- |
| NAVIS_HOST | Backend bind host | 0.0.0.0 for local network access |
| NAVIS_PORT | Backend port | 8000 |
| ESP32_CAMERA_URL | Camera stream or snapshot URL | Set from serial monitor / firmware output |
| YOLO_MODEL | Model weights path/name | Choose a small model and test locally |
| YOLO_CONFIDENCE | Detection threshold | Initial 0.55; tune using test data |
| FRAME_INTERVAL_MS | Minimum time between inferences | Start around 300–700 ms, benchmark laptop |
| ALERT_COOLDOWN_S | Repeat suppression interval | Start around 3–5 s, tune by scenario |
| LOG_LEVEL | Backend log verbosity | INFO |

# 6. API contracts

| Route | Method | Request | Response / behavior |
| --- | --- | --- | --- |
| /health | GET | None | {status, model_loaded, camera_configured} |
| /api/device | GET | None | {camera_connected, last_frame_at, backend_version} |
| /api/camera/snapshot | GET | Optional freshness flag | JPEG image or explicit camera error |
| /api/detect | POST | multipart image upload | {detections:[...], inference_ms, timestamp} |
| /api/analyze | POST | {mode:'describe'} | Concise grounded scene result or unavailable status |
| /api/ocr | POST | multipart image upload | {text, confidence/quality indicators, timestamp} |
| /api/command | POST | {command, text, location?} | {intent, status, message, data?} |
| /api/settings | GET | None | Current settings |
| /api/settings | PUT | Validated settings object | Updated settings |
| /api/navigation/route | POST | {destination, origin} | Route summary, maneuvers, distance and ETA if available |
| /api/navigation/progress | POST | {location, route_id} | Next maneuver and progress status |
| /ws/events | WebSocket | Client connects | Normalized event messages and status updates |

# 7. JSON schemas / examples

## 7.1 Detection event

{
  "event_id": "evt_001",
  "type": "detection",
  "timestamp": "ISO-8601 timestamp",
  "source": "camera",
  "objects": [{
    "label": "person",
    "confidence": 0.91,
    "bbox": [x1, y1, x2, y2],
    "position": "center"
  }],
  "distance_m": null,
  "priority": "normal",
  "message": "Person ahead"
}

Use distance_m=null until a calibrated ranging sensor is integrated. Position is an image-region estimate, not a world-coordinate measurement.

## 7.2 Command request and response

POST /api/command
{
  "command": "navigate",
  "text": "Take me to the library",
  "location": {"latitude": 0.0, "longitude": 0.0}
}

Response when destination is ambiguous:
{
  "intent": "navigate",
  "status": "destination_required",
  "message": "Which library do you mean?"
}

## 7.3 WebSocket event types

| Type | Required fields | Use |
| --- | --- | --- |
| hazard | event_id, timestamp, priority, message | Potentially relevant object alert. |
| navigation | timestamp, maneuver, distance_m, eta_s | Next route instruction; omit unknown fields. |
| status | timestamp, component, state, message | Camera/network/GPS/model status. |
| ocr_result | timestamp, text, quality | User-requested OCR output. |
| scene_result | timestamp, message, uncertainty | User-requested scene summary. |

# 8. Hazard engine rules

Reject malformed detections and detections older than the configured freshness window.

Apply a model confidence threshold; treat this as a tunable engineering parameter, not a safety guarantee.

Require temporal persistence or tracking for ordinary announcements; urgent classes may need a separately tested policy.

Deduplicate using class, approximate image region and a cooldown window.

Separate urgency from confidence: a low-confidence but potentially important detection may require a qualified message rather than silent dismissal.

Prioritize urgent alerts over ordinary narration and allow the user to cancel noncritical speech.

Do not infer physical distance from bounding-box area, confidence or class label.

If the camera, model or network is unavailable, publish a status event instead of stale detections.

# 9. ESP32-CAM setup and wiring

Use Espressif's official CameraWebServer example as the starting point.

Confirm the board is the common AI-Thinker ESP32-CAM before selecting CAMERA_MODEL_AI_THINKER; verify exact board revision.

Set Wi-Fi credentials locally and keep them out of version control.

If there is no onboard USB interface, use a compatible USB-to-serial adapter and follow its documented voltage/current requirements.

For typical AI-Thinker flashing mode, connect GPIO0 to GND while uploading; disconnect GPIO0 from GND and reset for normal boot.

Connect serial TX/RX crossed and share GND; power the board using a suitable regulated supply as specified by the board documentation.

Read the assigned IP from the serial monitor and test stream/snapshot in a browser before backend integration.

Do not assume an adapter's 3.3 V or 5 V output can supply sufficient current; unstable power commonly causes resets.

No ToF/ultrasonic sensor is currently included. Do not publish physical distance estimates in this build.

# 10. Android technical requirements

Use Kotlin and Jetpack Compose for UI.

Request microphone and location permissions at runtime with clear rationale and graceful denial behavior.

Use SpeechRecognizer for short commands and TextToSpeech for output; verify device language and offline/network behavior early.

Keep Android as the owner of speech output so route guidance and backend alerts share one prioritization queue.

Use a single network repository for REST and WebSocket operations.

Reconnect with bounded backoff; do not create infinite reconnect loops.

Use Android location APIs for coordinates and request only the permission level needed.

For route calculation, use a pedestrian-capable service and check its terms, rate limits, network needs and response schema.

Treat QR pairing as setup convenience, not authentication by itself. Validate any pairing token server-side and avoid placing secrets in the QR.

# 11. Networking and security

Laptop, Android and ESP32-CAM must be on a network that permits client-to-client traffic.

Bind FastAPI to 0.0.0.0 for LAN access, then configure Android with the laptop's LAN IP and port.

Do not expose the development server directly to the public internet.

Use local network only for the hackathon unless a cloud deployment is deliberately designed.

Validate image MIME type, payload size and command fields.

Avoid storing camera frames by default; logs should prefer timestamps, labels, confidence and latency over raw images.

Keep cloud API keys on the backend, not in the Android APK or firmware.

Use HTTPS and stronger authentication before any public or production deployment.

# 12. Module responsibilities and interfaces

| Module | Input | Output | Owner |
| --- | --- | --- | --- |
| Camera adapter | Stream URL | Timestamped frame | Embedded developer |
| Detection | Frame | Normalized detections | AI developer |
| Hazard engine | Detections + settings | Prioritized event | AI/integration |
| OCR | Frame on demand | Text + quality | AI developer |
| API/event manager | Module results | REST/WS messages | Integration lead |
| Voice interface | Speech text / event | Intent / spoken response | Android developer |
| Navigation | Destination + GPS | Route maneuvers and progress | Android developer |
| Settings | User changes | Validated persisted configuration | Integration + Android |
| Evaluation | Test cases and logs | Metrics and report | Research/testing lead |

# 13. Test strategy

| Test | Method | Record |
| --- | --- | --- |
| Camera reliability | Run stream for 5 minutes; note drops/reconnects | Frame success rate, reconnect time |
| Detection | Test common objects at different positions/lighting | Precision/recall on labelled subset |
| Alert filtering | Replay identical clips in baseline and filtered modes | Spoken alerts/minute, suppressed events |
| Latency | Timestamp frame, inference, event and speech start | Median and p95 latency |
| Voice commands | Repeat a fixed command set under controlled noise | Success rate and failure reason |
| OCR | Read clear, angled and blurred signs | Exact/character-level correctness where feasible |
| Navigation | Use a known short route in a safe area | Turn correctness, distance/ETA availability |
| Failure handling | Disconnect camera, stop backend, deny GPS permission | Correct status and recovery behavior |
| Privacy | Inspect logs and network destinations | No unintended raw-frame persistence or secret leakage |

# 14. Implementation milestones and exit gates

| Hours | Work | Exit gate |
| --- | --- | --- |
| 0–4 | Repo, environment, API schema, firmware/app scaffold | All projects start and owners agree on contracts. |
| 4–10 | Stream, YOLO, Android TTS | One real detection is spoken by Android. |
| 10–16 | Hazard engine, WebSocket, voice intents | Events flow end-to-end and duplicates are controlled. |
| 16–22 | GPS and route guidance | A known destination produces route guidance or a clear failure. |
| 22–27 | OCR, QR, settings | One P1 feature works and is integrated. |
| 27–32 | Reliability tests and metrics | Results and failure cases are logged. |
| 32–36 | Freeze, demo, documentation | Two rehearsals pass; no risky last-minute feature additions. |

# 15. Acceptance criteria

GET /health returns backend and model status.

ESP32-CAM frame is received and processed on the laptop.

YOLO results are normalized to a documented schema.

Android receives and speaks at least one real detection event.

Duplicate announcements are demonstrably reduced against a baseline.

Voice command repeat and cancel are available.

Navigation reports only valid route data and does not fabricate location or ETA.

Camera/network/GPS failures produce clear status messages.

No physical distance is claimed without a validated ranging sensor.

Setup, wiring, limitations, test results and demo steps are documented.

# 16. Risks and contingency paths

| Failure | Fallback |
| --- | --- |
| ESP32-CAM not stable by hour 8 | Use laptop webcam or prerecorded clips for AI integration; disclose this in demo and keep hardware stream as a separate milestone. |
| YOLO installation/model issue | Use a known compatible Python environment and small model; do not spend hours changing multiple dependencies simultaneously. |
| Android build is delayed | Use a minimal native screen with TTS and API calls before adding visual polish. |
| Route service fails | Use a pre-tested route fixture for UI integration, clearly label simulated route data, and retain live routing as a separately verified path. |
| OCR dependency conflict | Defer OCR; prioritize end-to-end safety/audio flow. |
| Cloud VLM unavailable | Use local detection summaries and state that rich scene description is unavailable. |

# 17. Definition of done and handoff

README includes setup, LAN IP configuration, run commands and troubleshooting.

All secrets and local credentials are excluded from Git.

API schemas and example messages match actual implementation.

Team can restart the firmware, backend and app from a clean state.

Evaluation logs include timestamped metrics and known limitations.

The demonstration is supervised and does not present NAVIS as a replacement mobility aid.

# 18. First implementation sprint: start here

Create the navis repository with firmware/, backend/, android/ and docs/ folders.

Member 1: flash CameraWebServer and send the camera IP/stream URL to the team.

Member 2: create Python 3.12 venv, start FastAPI /health and verify YOLO loads on a local image.

Member 3: create Android app with one button that speaks a test sentence using TextToSpeech.

Member 4: commit the API schema, coordinate the shared Wi-Fi setup and create a single integration checklist.

Integrate the first real frame and spoken detection before adding navigation, QR or OCR.